Worksheet on Thevenin Equivalent Circuits

Matt Trawick

Thevenin’s theorem says that if you have a circuit with two outputs that includes any combination of voltage sources and resistors (actually current sources too), then it is always functionally equivalent to a simpler circuit containing only one voltage source (VTH) and one internal resistance (RTH).  No electrical measurement can distinguish between the two. Thevenin’s theorem makes it easier to figure out how a complicated circuit will interact with other circuits or components, for instance if a load resistance is connected to the output terminals of the circuit below.

This bunch of crazy stuff…

…is equivalent to…

…this Thevenin equivalent circuit.

VTh

RTh

Here’s how you find the values of VTH and RTH.

Step 1: Find the open-circuit output voltage (the voltage with a very high load resistance).  This is VTH, known as the “Thevenin equivalent voltage.”

Step 2: Find the short-circuit output current ISHORT (the current with a very low load resistance).  The “Thevenin equivalent resistance” is given by RTH = VTH/ISHORT.

Step 2 (alternate): Imagine shorting out all of the voltage sources (and replacing any current source with an open), and find the resistance between the output terminals (with no additional RLOAD).  This is RTH.

Example problem: Consider the circuit drawn below.  Draw the Thevenin equivalent circuit, and find values for VTH and RTH.  (Step by step help follows….)

Step 1: What’s the voltage across the output leads (across the 2 kohm resistor) with no load attached?  This is VTH.

Step 2: What is the current shown on the ammeter below?  So what’s RTH?

A

Step 2 (alternate): With the voltage source short-circuited in the picture below, what is the resistance between the two output leads?

Answers: VTH=3.58 volts;  ISHORT=2.55 mA;  RTH=1.40 kΩ.