Physics 216: Electronics

“M” problems

Part C: Capacitors, Inductors, and AC circuits

Answers to problems are at the end of this document.

MC1.  A 200 nF capacitor is charged by a 5 V supply through a 2 kΩ resistor, starting at zero volts.  (a) Graph both the current IC through the capacitor and the voltage VC across the capacitor as functions of time.  (b) At what time does the capacitor voltage reach 90% of the supply voltage?  (c) What is the current through the resistor at the time you calculated in (b)?

MC2.  Consider the circuit shown below. (a) Calculate the two possible values of the input terminal V+, (the upper and lower threshold voltages) depending on the value of VOut.  You can assume R4≈0. (b) Draw graphs of voltage vs. time for Vout, V+, and V-, showing how they are synchronized.  (c) Calculate the time Δtrise for V- to rise from the lower threshold voltage to the higher threshold voltage.  (d) Calculate the time Δtfall for V- to fall from the higher threshold voltage to the lower threshold voltage.  (e) Find the output frequency of the oscillator.  Note: on an exam, you might be given this drawing and asked to find the frequency, without being asked all the helpful leading questions (a) through (d).

MC3.  The circuit to the right shows a voltage source with an internal impedance connected to a load via a transformer. (a) What is the resistance of the apparent load “seen” by the source? (b) What is the voltage across the transformer’s primary coil?  (c) What is the average power delivered to the load resistor, as shown?  (d) If you could replace the transformer, what turns ratio would you choose to deliver the maximum power to the load resistor?  (e) What is the maximum power you could deliver to the load?

MC4. Our goal in this problem is to analyze the circuit below, finding the voltage and current of each component.  We can’t tell ahead of time if there is current through the diode or not, so we’ll try both possibilities.

Assume for parts (a) through (d) that there is a forward current in the diode.

(a) Assuming there’s a forward current in the diode, what is the voltage VD across the diode?

(b) What are VR1 and VR2?

(c) What are IR1 and IR2?

(d) What is it about your last three answers that is inconsistent, thus demonstrating that our assumption in part (a) must be bogus?

Now assume that there is NO current in the diode.

(e) Assuming no current in the diode, does the diode behave like an open or like a short?

(f) Find VR1, VR2, and the current in the circuit.

(g) Are the voltages you found in part f consistent with your assumption in part e?

MC5. This problem is a lot like the previous one, except that you won’t be led through it step by step.  For the circuit below, find the voltages and currents for all resistors and the diode.

MC6. In this problem, the battery from the previous circuit has been replaced.  Find the voltages and currents for all resistors and the diode.

MC7. Do a little Googling for the 1N4001 diode you’ve been using in the lab, and find (a) its reverse breakdown voltage VR and (b) the maximum steady current you can pass through it in the forward direction, above which it presumably starts to smoke.  Cite your source or sources.  (I know I’ve already posted the datasheet for the 1N4001 online, but see if you can find the information on your own.)

MC8. Design two clipper circuits (series or shunt, your choice) for each of the following tasks: (a) The first circuit should accept an input of -10 V<Vin< +10 V and produce an output that is generally positive, clipping the negative part.  For this, you don’t particularly care whether the exact threshold for clipping is 0.0 or ±0.7 V, but you do care that for Vin above the threshold, Vin and Vout should differ by no more than 0.5%.  The load for this application will be about Rload= 50 kΩ.  (b) The second circuit should also accept an input of -10 V<Vin< +10 V, but produce an output that is generally negative, clipping the positive part.  You don’t care about the precision of the output much: either the exact clipping threshold or the exact ratio of Vout/Vin.  But this circuit will be used for Rload=50 Ω, drawing up to 200 mA.  Make sure that your circuit can handle this current.

MC9. The diagram below shows part of a circuit inside of an actual screw-in LED light bulb.  The AC voltage input Vin has already been reduced from 120 volts RMS to something smaller.  With the bulb turned on, the voltage across the capacitor is measured to have an average (DC) level of 28.7 volts and a ripple voltage of 3.4 volts.  (a) Rewrite your expression for ripple voltage you derived in the lab in terms of the maximum current Imax instead of Vmax.  (b) For small ripple voltages, Imax≈Iavg.  In fact, in your derivation, you used ΔQ=IΔt.  Which I is technically more accurate there, Iavg or Imax? (c) Based on the measurements given, find a numerical value for the average current through the LEDs.  (d) Find the total power dissipated by the LEDs (some of which, presumably, is in the form of visible light).  (e) Find the peak input voltage Vin required to make the bulb work as measured above.

MC10. This problem will step you through process of designing a 5-volt DC power supply that can supply up to 1 amp of current, with a ripple voltage of less than 2 mV.  You will use a full wave bridge rectifier and an MC7805 regulator, which has an output voltage of 5 V, a dropout voltage of 2 V, and a minimum ripple rejection of 62 dB.  (a) What is the minimum instantaneous input voltage for the regulator?  (b) What is the minimum ripple voltage at the input of the regulator to keep the output ripple from exceeding 2 mV?  (c) What value of capacitor is required to maintain that ripple voltage?  (d) What is the peak output voltage required from the transformer?  (e) Assume the AC line is nominally 120 volts RMS, but might drop as low as 105 volts.  What turns ratio transformer is required to ensure a stable 5 V output?

MC11. In the circuit below, a Zener shunt clipper is being used to regulate the voltage across Rload=1.6 kΩ.  The input voltage Vin varies sinusoidally between 7.5 and 9.0 volts.  (a) What value of Rlim is required for the circuit to function properly, and is that a minimum or a maximum value for Rlim?  (b) Assuming the value of Rlim from the last part is chosen, what is the average current through the diode?  (c) What is the maximum instantaneous power in both Rlim and the diode?

MC12. In the circuit below, the switch is closed at time t=0.  (a) Find the current IL through the inductor at t=10 msec.  (b) Is the current through the resistor IR the same as through the inductor at that time?  (c) Find the voltage across the inductor VL at that time.  Hint: you should have found in Lab 13 that the time constant for an “LR” circuit is given by τ=L/R.

MC13. This problem is analogous to the example we did in class, and for which I have posted an example problem on Blackboard.  Consider the circuit below, with a sinusoidal voltage source VSt=V0eiωt.  (a) Find the magnitude and phase of the current I(t) in the circuit.  (Both are functions of ω.)  (b) Find the magnitude and phase of the voltage VC across the capacitor.  (c) Find the "cutoff frequency" fC at which the ratio of  VC/V0=12, or about 0.707.  (d) At this frequency, what is VC/V0 expressed in decibels?  (e) Find expressions for VC/V0 in the limit of low frequency and in the limit of high frequency.  (f) Draw vectors on the complex plane for VS, VR, VC, and I.

MC14.  (True story!) A while ago I found some LED light bulbs for sale online that looked like a real bargain.  They were just the intensity I needed and they were super cheap, at a time when most LED bulbs were a lot pricier.  Unfortunately, after buying a bunch of them, I realized I had made a mistake: they were from China and were designed to work with 220 VAC, instead of 120 VAC like in the US.  D’oh!!!  I opened one up and found the circuit below, on the left.

The bulb uses what’s called a “drop cap” design, in which the capacitor C1 acts as a voltage divider with the rest of the circuit, yielding a reduced (“dropped”) voltage across the rectifier.  For this problem, we’ll make the gross simplification that the entire rest of the circuit acts like just an equivalent resistor REQ, as shown in the circuit on the right.  That way, you can treat the circuit using complex numbers, directly using your results from problem MC13.  (a) Given an input of V1=220 V RMS (Vpk=311.1 V), and a peak voltage of 31.8 V across the rectifier (or across REQ), what is the apparent value of REQ?  (b)  For the values in the previous part, what is the power dissipated in C1? (c) Now suppose that the bulb is to be used with an input voltage of V1=120 V RMS.  What new value of capacitor C1 is needed to maintain a peak voltage of 31.8 V across the rectifier? (d) In fact, I bought some larger capacitors, soldered them into the bulbs, and those LED bulbs remain in my house to this day, where they work just great!  Isn’t Matt clever for figuring this out?

MC15. For the circuit below, Vin=V0eiωt.  (a) Use the same techniques that you used in problem MC13 to derive algebraic expressions for the magnitude and phase of Vout.  (b) What is the very special value of ω (in terms of L and C) for which Vout is maximized? (c) What are the magnitude and phase of Vout/Vin for the three cases of (i) very low frequency, (ii) very high frequency, and (iii) ω=1LC?

MC16. In this problem you will analyze the circuit on the left, below, using the principle of superposition, which is also described in chapter 7 of Paynter.  (a) Imagine replacing the source V2 with a short, and calculate the voltage across R2.  (b) With source V2 restored, replace the source V1 with a short and calculate the voltage across R2.  (c) The actual voltage across R2 for the circuit as drawn below is the superposition of the two cases in (a) and (b).  So what’s VR2?

Shorting V2 in part (b)

Shorting V1 in part (a)

Original circuit

MC17. In this problem you will again analyze a circuit (below) using the principle of superposition, and those of you with a flair for observation will notice a certain similarity with the previous problem.  (a) Imagine replacing the source V2 with a short, and calculate the voltage Vout, assuming the circuit has been on long enough that the capacitor is fully charged.  (b) With source V2 restored, replace the source V1 with a short.  What kind of filter is the resulting circuit, and what is its cutoff frequency?  With V1 shorted out, what is Vout in the limit of both high and low frequencies?  (c) The actual voltage Vout for the circuit as drawn below is the superposition of the two cases in (a) and (b).  So what is Vout, in the limit of both high and low frequencies?

Redrawn to look exactly like MC16.

MC18: For the circuit below (from Paynter, page 448, problem 38), use Mathematica as a complex number calculator as we did in class to find the magnitude and phase of the voltage across capacitor C2.

MC19: In the circuit below, the Zener diode has VF=0.7 V and VR=3.2 volts. (a) Find the instantaneous current through resistor R1 at the time of the peak positive source voltage (+5 volts). (b) Find the instantaneous current through resistor R1 at the time of the peak negative source voltage (-5 volts).  Hint: One way to analyze the circuit in the case that you assume you know Vdiode is to treat the diode as a known voltage source (a battery) and use the principle of superposition as you did in problem MC16.

MC20: In problem MC13, you found that for a low-pass RC filter,

VoutVin=1ωRC2+1  .

(a) Rewrite this expression in terms of the frequency f and the cutoff frequency fc.  (b) Suppose you have just designed a part of a circuit that gives a sinusoidal output signal at 100 Hz.  Unfortunately, you realize that the output voltage also includes some interference at 25 kHz from a nearby video scanner circuit, which you decide to reduce by applying a low-pass RC filter to your output.  What cutoff frequency fc should you choose for your filter to reduce the interference by a factor of about 100, while still allowing at least 90% of your signal to pass?

MC21: The diagram below shows a clipper circuit with two Zener diodes.  (a) Draw a graph of the output voltage vs. time, labeling the minimum and maximum values. (b) Calculate the maximum magnitude of the instantaneous current through the diodes. (c) Find the maximum instantaneous power dissipated in diode D2 and in the resistor Rlim.

Answers:

MC1. (b) 921 μsec (c) 250 μA

MC2. (a) 2.974 and 2.026 volts (c) 27.9 μsec (d) also 27.9 μsec (e) 17.92 kHz

MC3. (a) 144 Ω (b) 4.64 volts RMS (c) 150 mW (d) 6.12:1 (e) 240 mW

MC4. (a) 2.2 V (b) 2.8 V and 2.2 V (c) 2.8 mA and 5.5 mA (d) We can’t have IR2>IR1.  If we did, that would mean that negative current flows through the diode, that is, flowing in the reverse direction.  That directly contradicts the assumption that the diode has a forward current. (e) open (f) VR1=3.57 V; VR2=1.43 V; ID=0.

MC5. VR1=2.2 V;VR2=3.1 V;VR3=6.7 V;IR1=IR3=2.2 mA;IR2=1.6 mA;ID=0.6 mA

MC6. VR1=1.33 V;VR2=2.67 V;VR3=4.0 V;IR1=IR2=IR3=1.33 mA;ID=0

MC9. (b) Iavg  (c) 19 mA (d) 0.55 Watts (e) 31.8 V

MC10. (a) 7 V (b) 2.52 V (c) 3300 μF (d) 10.92 V (e) maximum of 13.6:1

MC11. (a) Max value of Rlim is 800 Ω. (b) 938 μA (c) 20 mW and 9.38 mW, respectively

MC12. (a) 28.2 mA (b) Yes, they are always the same; otherwise components would build up static charges and start attracting each other with large Coulomb forces, which would be dangerous and embarrassing. (c) 2.68 V

MC13. (a) I=V0R2+1ωC2 ; ϕ=tan-11ωRC  (b) VC=V0ωRC2+1;                      ϕ=-tan-1ωRC (c) fC=12πRC (d) about -3 dB (e) As ω→0, VC/V0=1. As ω→∞, VC/V0≈ωRC-1.

MC14. (a) 332.4 Ω  Hint: use your expression for I in MC13 to find an expression for VR.    (b) Zero.  That’s why you do this with a capacitor instead of a resistor. (c) 1.52 μF  (d) No, Matt is a dumbass for buying the wrong bulbs in the first place.

MC15. (a) VC=V01+R21ωL-ωC2; ϕ=tan-1R1ωL-ωC  		        (c) (i): VoutVin→ωLR; ϕ→+90° (ii): VoutVin→1ωRC; ϕ→-90°               (iii): VoutVin=1; ϕ=0°

MC16. (a) 3 V (b) 1.5 V (c) 4.5 V

MC17. (a) 6 V DC (b) High pass, fc=318 Hz (c) f≳fc:Vout=V2+6 V;f≪fC:Vout≈6 V

MC18. 3.719 V; -34.32°.  (Is your answer for the magnitude off by a factor of 2? Remember that the “6 volts” on the source is an RMS value….)

MC19. (a) 2.5 mA (b) -3.1 mA.  Hint: if you got 2.27 mA for part (a), you probably assumed that there is current through the diode, so that the voltage across it is 3.2 V.  But as you learned in problem MC4, it’s also possible that there is no current through the diode.  One of these scenarios leads to a physically plausible solution, and the other does not; you’ll need to try both to see which one is correct.

MC20. Anything between fc=206 Hz and fc=250 Hz will work fine.

MC21. (b) 5.6 mA (c) 18.5 mW and 36 mW.