MC10.  This problem will step you through process of designing a 5 volt DC power supply that can supply up to 1 amp of current, with a ripple voltage of less than 2 mV.  You will use a full wave bridge rectifier and an MC7805 regulator, which has an output voltage of 5 V, a dropout voltage of 2 V, and a minimum ripple rejection of 62 dB.  (a) What is the minimum instantaneous input voltage for the regulator?  (b) What is the minimum ripple voltage at the input of the regulator to keep the output ripple from exceeding 2 mV?  (c) What value of capacitor is required to maintain that ripple voltage?  (d) What is the peak output voltage required from the transformer?  (e) Assume the AC line is nominally 120 volts RMS, but might drop as low as 105 volts.  What turns ratio transformer is required to ensure a stable 5 V output?

MC10. (a) 7 V (b) 2.52 V (c) 33,000 μF (d) 10.92 V (e) 13.6:1