ME5 solutions:

- The three resistors provide a connection between the flip-flop inputs and ground, which assures that the inputs will be read as “0” when the buttons are not pressed.  Remember that an open digital input is NOT guaranteed to be read as a “0”, and is frequently read as a logical “1”.

- If player 1 hits the button:  S1=1, Q1=1,  Q1=0, and R2=1.

- When player 1 releases the button, S1 goes back to 0, but everything else stays the same as in part (b).

- Per the figure below, if player 2 hits the button after player 1, then the input R2=1 already, due to the output from Q1.  The button causes S2=1, which changes Q2=1, but leaves Q2=1 unchanged.

S

R

Q

_Q

S

R

Q

_Q

inputs

time

outputs

time

Q and Q normally have opposite values…

_

…but Q and Q are both high when R and S are both high, regardless of sequencing.

_

- As part (d) shows, if player 1 has already hit their button, then player 2 hitting their button later causes Q2=1.  If the light LED2 depended on Q2 directly, then player 2’s light would come on as long as their button remained pressed—causing both players’ lights to be on simultaneously and confusing the host.  By connecting LED2 to NOT(Q2), we ensure that the two players’ lights are never on at the same time.

- In flip-flop U1, S1=R1=0, meaning the clock and data will affect it.  And because DA1=0, the rising CL1 from the host button sets Q1=0 and Q1=1, turning LED1 off.

In flip-flop U2, R2=1, meaning that CL2 and DA2 have no effect.  However, player2’s light is already off.  And resetting Q1=0 causes R2=S2=0, leaving U2 ready to respond if player 2 hits their button first next time.