MC10.  (True story!) A while ago I found some LED light bulbs for sale online that looked like a real bargain.  They were just the intensity I needed and they were super cheap, at a time when most LED bulbs were a lot pricier.  Unfortunately, after buying a bunch of them, I realized I had made a mistake: they were from China and were designed to work with 220 VAC, instead of 120 VAC like in the US.  D’oh!!!  I opened one up and found the circuit below, on the left.

The bulb uses what’s called a “drop cap” design, in which the capacitor C1 acts as a voltage divider with the rest of the circuit, yielding a reduced (“dropped”) voltage across the rectifier.  For this problem, we’ll make the gross simplification that the entire rest of the circuit acts like just an equivalent resistor REQ, as shown in the circuit on the right.  (a) Given an input of V1=220 V RMS, and a peak voltage of 31.8 V across the rectifier (or across REQ), what is the apparent value of REQ?  (b)  For the values in the previous part, what is the power dissipated in C1? (c) Now suppose that the bulb is to be used with an input voltage of V1=120 V RMS.  What new value of capacitor C1 is needed to maintain a peak voltage of 31.8 V across the rectifier? (d) In fact, I bought some larger capacitors, soldered them into the bulbs, and those LED bulbs are now all in my house where they work just great.  Isn’t Matt clever for figuring this out?

MC10. (a) 332.4 Ω  (b) Zero.  That’s why you do this with a capacitor instead of a resistor. (c) 1.52 μF  (d) No, Matt is a dumbass for buying the wrong bulbs in the first place.